IoT Based Air Pollution Monitoring System

A Project report submitted for IBM

By

S.vaishali

TABLE OF CONTENTS

T OPICS	P AGE NO.

List of figures	i

List of acronyms	ii

ABSTRACT	iii

CHAPTER 1: Introduction	01-02

  - Aim	01

  - Literature Survey	01-02

CHAPTER 2: Theory & Description of the Components	03-10

  - What is IoT?	03-04

  - Components Used	05

  - Brief Description of the Components	05-10

  - Working Procedures	10

CHAPTER 3: Hardware Model	11-14

  - Hardware Model to Preheat DHT11 Sensor Module	11

  - Hardware Model to Preheat and Calibrate MQ-135 Gas Sensor Module	12-13

  - Final Hardware Model	13-14

CHAPTER 4: Algorithm and Software Program	15-21

  - Working Algorithm	15

  - Calibration of MQ-135 Gas Sensor Module	16-19

  - Execution of the Main Program	19-21

CHAPTER 5: Results	22-28

CHAPTER 6: Conclusion	29

REFERENCES	30

APPENDIX

  - Pin Description of NodeMCU	31-32

  - Description of Software Libraries used	33

  - Cost Estimation	34

### L ist of Figures

Figure No.	 Figure Title

1	IoT based air pollution monitoring system

2.1	Pinout Diagram of NodeMCU V3

  - Pinout Diagram of DHT11 sensor

  - The structure of the humidity sensor

  - Mq-135 Gas Sensor Module

  - Veroboard

  - AC-DC Adapter

  - LEDs

  - Resistors

  - Arduino IDE

  - ThingSpeak Cloud

  - Circuit Diagram to Pre-heat the DHT11 sensor module

  - Circuit Diagram to Pre-heat the MQ-135 gas sensor module

  - Circuit Diagram to Calibrate the MQ-135 Gas Sensor module

  - Circuit Diagram of the setup

  - Observations for Experiment 1

  - Setup for Experiment 1

  - Observations for Experiment 2

  - Setup for Experiment 2

  - Observations for Experiment 3

  - Setup for Experiment 3

  - Setup for Experiment 4

  - Observations for Experiment 4

  - Setup for Experiment 5

  - Observations for Experiment 5

### L ist of Acronyms

DHT	Digital Humidity and Temperature IoT		Internet of Things

PPM	Parts Per Molecule

PM	Particulate Matter

CO	Carbon Monoxide

CO2	Carbon Dioxide

LED	Light Emitting Diode

LPG	Liquid Petroleum Gas

IDE	Integrated Development Environment

ABSTRACT

Air pollution is one of the biggest threats to the present-day environment.  Everyone  is  being  affected by air pollution day by day including humans, animals, crops, cities, forests and aquatic ecosystems. Besides that, it should be controlled at a certain level to prevent the increasing rate of global warming. This project aims to design an IOT-based  air pollution monitoring system  using the internet from anywhere using a computer or mobile to monitor the air quality of the surroundings and environment. There are various methods and instruments available for the measurement and monitoring quality of air. The IoT-based air  pollution  monitoring  system  would not only help us to monitor the air quality but also be able to send alert signals whenever the air quality deteriorates and goes down beyond a certain level.

In this system, NodeMCU plays the main controlling role. It has been programmed in a manner,  such that, it senses the sensory signals from the sensors and shows the quality level via led

indicators. Besides the harmful gases (such as CO2, CO, smoke, etc) temperature and humidity  can  be monitored through the temperature and humidity sensor by this system. Sensor responses are fed  to the NodeMCU which displays the monitored data in the ThingSpeak cloud which can be utilized for analyzing the air quality of that area. The following simple flow diagram (as shown in Fig. 1) indicates the working mechanism of the IoT-based Air Pollution Monitoring System.

Fig.1. IoT based Air Pollution Monitoring System

## Chapter1 INTRODUCTION

#### Aim of the Project

Air is getting polluted because of the release of  toxic  gases  by  industries,  vehicle  emissions and increased concentration of harmful gases and particulate matter in the atmosphere.

The level of pollution is increasing rapidly due to factors like industries, urbanization,  increase in population, vehicle use which  can  affect  human  health.  Particulate  matter  is one of the most important parameters having  a  significant  contribution  to  the  increase in air pollution. This creates a need for measurement and analysis of real-time air quality monitoring so that appropriate decisions can be taken in a timely period.

This paper presents real-time standalone air quality monitoring. Internet of Things (IoT)is nowadays finding profound use in each and every sector, plays a key role in our air quality monitoring system too. The setup will show the air quality in PPM  on the webpage  so that  we can monitor it very easily.

In this IoT project, we  can monitor the pollution level  from anywhere using your computer  or mobile.

#### Literature Survey

The explanation of the Air Quality Index  (AQI) and  its  standard  ranges are  described in  [1]. From 0-100 ppm the atmosphere is safe for  living.  If  the  ppm  level increases  above 100 then it moves out of the safety zone. If the ppm value rises above 200 then it becomes extremely dangerous for human life.

The DHT11 sensor module is used to measure the temperature and the humidity of the surroundings [2]. The MQ-135 gas sensor is used to measure the air quality of the surroundings [3]. It can be calibrated with respect to fresh air, alcohol, carbon dioxide,

hydrogen and methane. In this project, it has been calibrated with respect  to fresh  air [9],  [10].

In [4] the controlling action of NodeMCU has been described. This research has shown the uses of C++ as the programming language for scripting the  software  code.  It  has  an in-  built Wi-Fi module which allows the project to  implement  IoT  easily. Arduino IDE  is used to implement the coding part of the project [5], [8]. ThingSpeak cloud is used for the cloud service. It has a free version which requires a delay of 15 seconds to upload an entry in the cloud [6], [7]. As this project uses two sensors, both of them have internal heater elements  and withdraw more power(P=V*I), so though both sensors are turned ON, their  output voltage levels vary and show unpredictable values due to insufficient power drive. So,  we used a separate power supply for the sensors as NodeMCU alone is not sufficient  to  drive  two sensors [9].

## Chapter 2

THEORY & DESCRIPTION OF THE COMPONENTS

#### What is IOT?

The Internet of Things (IoT) describes the network of physical objects—“things”—that are embedded with sensors, software, and other technologies for the purpose of connecting and exchanging data with other devices and systems over the internet. These devices range from ordinary household objects to sophisticated industrial tools.

The field has evolved due to the convergence of multiple technologies, including ubiquitous computing, commodity sensors, increasingly powerful embedded systems, and machine learning.

Traditional fields of embedded systems, wireless sensor  networks,  control  systems, automation (including home and building automation), independently and collectively enable the Internet of things. In the consumer market, IoT technology is most synonymous with products pertaining to the concept of the "smart home",  including devices  and  appliances (such as lighting fixtures, thermostats, home security systems, cameras, and other home appliances) that support one or more common ecosystems, and can be controlled via devices associated with that ecosystem, such as smartphones and smart speakers. IoT is also used in healthcare systems.

There are a number of concerns about the risks in the growth of IoT technologies  and  products, especially in the areas of privacy and security, and consequently, industry and governmental moves to address these concerns have begun, including the development of international and local standards, guidelines, and regulatory frameworks.

IoT devices are a part of the larger concept of home automation, which can include lighting, heating and air conditioning, media and security systems and camera systems. Long-term benefits could include energy savings by automatically ensuring lights and electronics are turned off or by making the residents in the home aware of usage.

A smart toilet seat that measures blood pressure, weight, pulse and oxygen levels. A smart home or automated home could be based on a platform or hubs that control smart devices and appliances. For instance, using Apple's HomeKit, manufacturers can  have  their  home  products and accessories controlled by an application in iOS  devices such as the iPhone and  the Apple Watch. This could be a dedicated app or iOS native applications such as Siri. This

can be demonstrated in the case of Lenovo's Smart Home Essentials, which is a line of smart home devices that are controlled through Apple's Home app or Siri without the need for a  Wi-Fi bridge. There are also dedicated smart home hubs that are offered as standalone platforms to connect different smart home products and these include the Amazon Echo, Google Home, Apple's HomePod, and Samsung's SmartThings Hub. In addition to the commercial systems, there are many non-proprietary, open-source ecosystems; including  Home Assistant, OpenHAB and Domoticz.

Significant numbers of energy-consuming devices (e.g. lamps, household appliances, motors, pumps, etc.) already integrate Internet connectivity, which can allow them to communicate  with utilities not only to balance power generation but also helps optimize the energy consumption as a whole.These devices allow for remote control by users, or central management via a cloud-based interface, and enable functions like scheduling (e.g., remotely powering on or off heating systems, controlling ovens, changing lighting conditions, etc.).The smart grid is a utility-side IoT application; systems gather and act on energy  and power-  related information to improve the efficiency of the production and distribution of electricity.Usingadvanced metering infrastructure (AMI) Internet-connected devices, electric utilities not only collect data from end-users but also manage distribution automation devices like transformers.

Another example of integrating the IoT is Living Lab which integrates and combines research and innovation processes,  establishing  a  public-private-people-partnership.There  are currently 320 Living Labs that use the IoT to collaborate and share knowledge between stakeholders to co-create innovative and technological products. For companies to implement and develop IoT services for smart cities, they need to have incentives. The governments play key roles in smart city projects as changes in policies will help cities to implement the IoT which provides effectiveness, efficiency, and accuracy  of the resources that are being used.  For instance, the government provides tax incentives and cheap rent, improves  public  transport, and offers an environment where start-up companies, creative industries, and multinationals may co-create, share a common infrastructure and labor markets, and take advantage of locally embedded technologies, production process, and transaction costs.The relationship between the technology developers and governments who manage  the  city's  assets is key to providing open access to resources to users in an efficient way.

In this project, we have tried to implement the concept of IoT to monitor the temperature, humidity and air quality of the surroundings

#### Components Used

######  Hardware Components

    - NodeMCU V3

    - DHT11 Sensor Module

    - MQ-135 Gas Sensor Module

    - Veroboard(KS100)

    - Breadboard

    - Connecting Wires

    - AC-DC Adapters

    - LEDs emitting green, yellow and red colours

    - Resistors

######  SOFTWARE COMPONENTS

- ThinkSpeak Cloud

- Arduino IDE

#### Brief Description of the Components

######  NodeMCU V3

NodeMCU V3 is an open-source ESP8266 development kit, armed with the CH340G USB- TTL Serial chip. It has firmware that runs on ESP8266 Wi-Fi SoC from Espressif Systems. Whilst cheaper, CH340 is super reliable even in industrial applications. It  is  tested  to  be stable on all supported platforms as well. It can  be simply coded in Arduino  IDE.  It has a  very low current consumption between 15 µA to 400 mA.

The pinout Diagram of NodeMC3 is shown in Fig. 2.1.

Fig. 2.1 (Pinout Diagram of NodeMCU V3)

######  DHT11 Sensor Module

The DHT11 is a temperature and humidity sensor that gives digital output in terms of voltage. It uses a capacitive humidity sensor and a thermistor to measure the surrounding air.

As shown in Fig. 2.2, we need to supply a voltage of 5V (DC) to the Vcc pin and ground it to the GND pin. The sensor output can be easily read from the Data pin in terms of voltage (in digital mode).

Humidity Measurement: The humidity sensing capacitor has two electrodes with a moisture-holding substrate as a

Fig 2.2 (Pinout Diagram of DHT11sensor)

dielectric between them as shown in Fig 2.3. Change in the capacitance value occurs with the change in humidity levels. The IC measure, process these changed resistance values and then converts them into digital form.

Temperature Measurement: For measuring the temperature, the  DHT11  sensor  uses  a negative temperature coefficient thermistor, which causes a decrease in its resistance value  with an increase in  temperature. To get a wide range of resistance values, the sensor is made  up of semiconductor ceramics or polymers.

Fig 2.3(The structure of the humidity sensor)

######  MQ-135 Gas Sensor Module

The material of MQ135 is SnO2, it is a special material: when exposed  to clean air, it is   hardly being conducted, however, when put in an environment with combustible gas, it has a

pretty performance of conductivity. Just make a simple electronic circuit, and convert the change of conductivity to a corresponding output signal. MQ135 gas sensor is sensitive to Ammonia, Sulphide, Benzene steam, smoke and other harmful gases. Used for family, surrounding environment noxious gas detection device,  apply  to  ammonia,  aromatics, sulphur, benzene vapor, and other harmful gases/smoke, gas detection, tested concentration range: 10 to 1000ppm. In a normal environment, the environment  which  doesn’t  have detected gas set the sensor’s output voltage as the reference voltage,  the  analog  output  voltage will be about 1V, when the sensor detects gas, harmful gas concentration increases by 20ppm per voltage increase by 0.1V.

Fig 2.4 (MQ-135 Gas Sensor Module)

######  Veroboard (KS100)

Veroboard is the original prototyping board. Sometimes referred to as ‘stripboard’ or ‘matrix board’ these offer total flexibility for hard wiring discrete components. Manufactured  from  a copper clad laminate board or Epoxy based substrate, it is offered in both single and double- sided formats. Vero boards are available  in  a wide range of board sizes and in both imperial  and metric pitch – Veroboard is an ideal base for circuit construction and offers even greater

adaptability using our range of terminal pins and assemblies. As with other stripboards, in using

Fig 2.5 Veroboard

Veroboard, components are suitably positioned and soldered to the conductors to form the required circuit. Breaks can be made in the tracks, usually around holes,  to divide the strips  into multiple electrical nodes enabling increased circuit complexity. This type of wiring

board may be used for initial electronic circuit development, to  construct  prototypes  for  bench testing or in the production of complete electronic units in small quantities.

######  AC-DC Power Adapter

An AC-DC power supply or adapter is an electrical device that obtains electricity from a grid-based power supply and converts it into a

different current, frequency, and voltage. AC-DC power supplies are necessary to provide the right power that an electrical component needs. The AC- DC  power  supply  delivers  electricity  to  devices that

would typically run-on batteries or have no other power source.

Fig 2.6 AC-DC Power Adapter

######  LED (Red, Green & Yellow)

A light-emitting diode (LED) is a semiconductor light source that emits light when current flows through it. Electrons in the semiconductor recombine with electron holes, releasing energy in the form of photons. The colour of the light (corresponding to the energy of the photons) is determined by the energy required for electrons to cross the band gap of the semiconductor. White light is obtained by using multiple semiconductors or a layer of light- emitting phosphor on the semiconductor device. LEDs have many advantages over incandescent light sources, including lower power consumption, longer lifetime, improved physical robustness, smaller size, and faster switching. In exchange for these generally favourable attributes, disadvantages of LEDs

include electrical limitations to low voltage and generally to DC (not AC) power, inability to provide steady illumination from a pulsing DC  or an AC electrical supply source, and lesser maximum operating temperature and storage temperature.  In  contrast  to  LEDs, incandescent

lamps can be made to intrinsically run at virtually

any supply voltage, can utilize either AC or DC

Fig 2.7LEDs

current interchangeably, and will provide steady illumination when powered  by  AC  or  pulsing DC even at a frequency as low as 50 Hz. LEDs usually need electronic support

components to function, while an incandescent bulb can and usually does  operate  directly  from an unregulated DC or AC power source.

######  Resistors

A resistor is a passive two-terminal electrical component that implements electrical resistance as   a circuit element. In electronic circuits, resistors are used to reduce current flow, adjust signal levels, to divide voltages, bias active elements, and terminate transmission lines, among other uses. High-power resistors that can dissipate many watts of electrical power as heat may be used as part of motor

controls, in power distribution systems, or as test  loads for generators. Fixed resistors have resistances

Fig 2.8 Resistors

that only change slightly with temperature, time or operating voltage.

######  Arduino IDE

The Arduino IDE is open-source software, which is used to write and upload code to the Arduino boards. The IDE application is suitable for different operating systems such as Windows, Mac OS X, and Linux. It supports the programming languages C and C++.	Here,	IDE		stands	for Integrated		Development Environment. The program or

Fig 2.9Arduino IDE

code written in the Arduino IDE is often called sketching. We need to connect the  Genuino  and Arduino board with  the IDE to upload the sketch written in the Arduino IDE software.  The sketch is saved with the extension '.ino.'

######  ThingSpeak Cloud

Thing Speak is open-source software  written in Ruby which allows users to communicate with internet-enabled devices. It facilitates data access, retrieval and logging of data by providing an API to both the devices and social network websites. Thing Speak was originally launched by iot Bridge in 2010 as a service in support of IoT applications. Thing Speak has integrated support from the numerical computing software MATLAB from Math Works, allowing Thing Speak users

Fig 2.10 ThingSpeak Cloud

to analyse and visualize uploaded data using MATLAB without requiring the purchase of a MATLAB license from Math Works.

#### Working Procedures

NodeMCU plays the main controlling role in this project. It  has been programmed  in a manner,  such that, it senses the sensory signals from the sensors and shows the quality level via  led  indicators. The DHT11 sensor module is used to measure the temperature and the humidity of the surroundings. With the help of the MQ-135 gas sensor module, air quality is measured  in  ppm.  These data are fed to the Think Speak cloud over the internet. We have  also  provided  LED  indicators to indicate the safety levels.

STEP 1.	Firstly, the calibration of the MQ-135 gas sensor module is done. The sensor is set to preheat for 24 minutes. Then the software code is uploaded to the NodeMCU followed by the hardware circuit to calibrate the sensor has been performed.

STEP 2.	Then, the DHT11 sensor is set to preheat for 10 minutes.

STEP 3.	The result of calibration found in STEP 1 is used to configure the final  working code.

STEP 4.	The final working code is then uploaded to the NodeMCU. STEP 5.	Finally, the complete hardware circuit is implemented.

The software codes and the hardware circuits are described in the following chapters.

# Chapter 3 HARDWARE MODEL

#### Hardware Model to Preheat DHT11 Sensor Module

As discussed earlier, we need to preheat the DHT11 sensor so that it can work accurately. The following steps were performed to preheat the DHT11 sensor module:

STEP 1 :	The Vcc  pin of the DHT11 sensor module was connected with the VU pin of NodeMCU.

STEP 2 :	The Gnd  pin of the DHT11 sensor module was connected with the Gnd pin of NodeMCU.

STEP 3 :	The NodeMCU is powered with a 12V DC via AC-DC adapter for 20 minutes. STEP 4 :	The setup was then disconnected.

Fig. 3.1 shown below describes the foresaid connections.

Fig. 3.1 (Circuit Diagram to Preheat the DHT11 sensor module)

#### Hardware Model to Preheat and Calibrate MQ-135 Gas Sensor Module

The following steps were performed to preheat the MQ-135 gas sensor module

STEP 1 : The Vcc pin of the MQ-135 gas sensor module was connected with the VU pin of NodeMCU.

STEP 2 : The Gnd  pin of the MQ-135 gas sensor module was connected with the Gnd pin of NodeMCU.

STEP 3 : The NodeMCU is powered with a 12V DC via AC-DC adapter for a day.

STEP 4 : The setup was then disconnected.

Fig. 3.2 shown below describes the foresaid connections.

Fig. 3.2(Circuit Diagram to Preheat the MQ-135 Gas sensor module)

The following steps were performed to calibrate the MQ-135 gas sensor module

STEP 1 : The Vcc pin of the MQ-135 gas sensor module was connected with the VU pin of NodeMCU.

STEP 2 : The Gnd pin of the MQ-135 gas sensor module was connected with the Gnd pin of NodeMCU.

STEP 3 : The analog DATA pin of the MQ-135 gas sensor module was connected with the A0 Pin of the NodeMCU.

STEP 4 : The software code to calibrate the sensor is then uploaded to the NodeMCU and the value of R0in fresh air is collected from the serial monitor of the Arduino IDE.

STEP 5 : The setup was then disconnected.

Fig. 3.3 shown below describes the foresaid connections.

Fig. 3.3(Circuit Diagram to Calibrate the MQ-135 Gas sensor module)

#### Final Hardware Model

The following steps were performed to execute the project

STEP 1 : The Vcc pin of the MQ-135 gas sensor module and DHT11 sensor module was connected via Veroboard with an adapter delivering around 5V.

STEP 2 : The Gnd pin of the MQ-135 gas sensor module, DHT11 sensor module and the cathode of the LED indicators was connected via Veroboard with the Gnd pin of the NodeMCU.

STEP 3 : The analog DATA pin of the MQ-135 gas sensor module was connected with the A0 Pin of the NodeMCU.

STEP 4 : The DATA pin of the DHT11 sensor module was connected with the D0 pin of the NodeMCU.

STEP 5 : The anode of the three LED indicators (green, yellow, and red) were connected to the D2, D3, and D4 pins of the NodeMCU respectively.

STEP 6 : The software code to execute the project was then uploaded to the NodeMCU. STEP 7 : The setup was then powered with 9V DC via AC-DC adapter.

It can be now turned ON/OFF as per the requirements. Fig 3.4 represents the circuit diagram of the setup.

Fig. 3.4(Circuit Diagram of the setup)

# Chapter 4

SOFTWARE IMPLEMENTATION

#### Working Algorithm

START

Turn ON the Power Switch

Read the sensed values from DHT11 sensor module and MQ-135 Gas Sesnsor Module

Send the sensed data in the ThinkSpeak Channel in respective fields

If ppm value

<100

No

Yes

If ppm value

<200

No

Turn on the Red LED and Turn OFF the Green and Yellow LEDs

Yes

ON

If power switch

OFF

END

Turn on the Yellow LED and Turn OFF the Red and Green LEDs

Turn on the Green LED and Turn OFF the Red and Yellow LEDs

##### Calibration of MQ-135 Gas Sensor Module

######  Theory of Calibration [10]

The most important step is to calibrate the sensor in the fresh air and then draw an  equation  that converts the sensor output voltage value into our convenient units PPM (parts  per  million). Here are the mathematical calculations derived,

Fig. 4.1 (Internal Circuit diagram of MQ-135 sensor)

From Ohm’s Law, at a constant temperature, we can derive I as follows:

I = V R

(1)

From Fig 4.1, eqn. 1 is equivalent to

I =	Vc

Rs+Rl

(2)

From Fig 4.1, we can obtain the output  voltage at  the load resistor using the value obtained   for I and Ohm’s Law at a constant temperature, V = I x R.

VRL = [ VC/ (RS + RL)] x RL	(3)

VRL = [(VC∗ RL)/(RS + RL)]	(4)

So now we solve for RS:

VRLx (RS + RL) = VCx RL	(5)

(VRLx RS) + (VRL x RL) = VC  x RL	(6)

VRLx RS = (VC∗ RL) − (VRL∗RL)	(7)

RS = {(VC∗ RL − (VRL∗ RL)} / VRL	(8)

RS = {(VC∗ RL) VRL} – RL	(9)

Eqn. 9 helps us to find the internal sensor resistance for fresh air.

Fig. 4.2 (Graph representing ratio vs ppm variations)

From the graph shown in fig 4.2, we can see that the resistance ratio in fresh air is a constant:

RS / R0 = 3.6	(10)

Value 3.6 which is mentioned in eqn. 10 is depicted in the datasheet shown in Fig 4.2. To calculate R0, we will need to find the value of the RS in the fresh air. This will be done by taking the analog average readings from the sensor and converting them to voltage. Then we will use the RS formula to find R0. First of all, we will treat the lines as if they were linear.  This way we can use one formula that linearly  relates the ratio and the concentration. By   doing so, we can find the concentration of a gas at any ratio value even outside of the graph’s boundaries. The formula we will be using is the equation for a line, but for a log-log scale. From above Figure 4.2, we try to derive the following calculations.

y = mx + b	(11)

For a log-log scale, the formula looks like this:

log10y = m ∗ log10x + b	(12)

Let’s find the slope. To do so, we need to choose 2 points from the graph. In our case, we  chose the points (200,2.6) and (10000,0.75). The formula to calculate slope m(here) is the following:

m =  {logy  −  log(y0)} / { logx  − log(x0)}	(13) If we apply the logarithmic quotient rule, we get the following:

m = log(

y ) / log( x )	(14)

yO	sO

Now we substitute the values for x, x0, y, and y0:

m = log(0.75/2.6) / log(10000/200)	(15)

m = −0.318	(16)

Now that we have m, we can calculate the y-intercept. To do so, we need to choose one point from the graph (once again from the CO2 line). In our case, we chose (5000,0.9)

log(y) = m ∗ log(x) + b	(17)

b = log(0.9) − (−0.318) ∗ log(5000)	(18)

b = 1.13	(19)

Now that we have m and b, we can find the gas concentration for any ratio with the following formula:

log(x) = {log(y) − b} / m	(20)

However, in order to get the real value of the gas concentration according to the log-log plot we need to find the inverse log of x:

x = 10 ^ [{log(y)−b] / m	(21)

Using eqns. 9 and 21, we will be able to convert the sensor output values into PPM (Parts per Million). Now we developed the Code and flashed into the NodeMCU giving proper connections.

######  SOFTWARE CODE for Calibration of MQ135 Sensor:

void setup()

{

Serial.begin(9600); //Baud rate

pinMode(A0,INPUT);

}

void loop()

{

float sensor_volt; //Define variable for sensor voltage float RS_air; //Define variable for sensor resistance float R0; //Define variable for R0

float sensorValue=0.0; //Define variable for analog readings Serial.print("Sensor Reading = "); Serial.println(analogRead(A0));

for(int x = 0 ; x < 500 ; x++) //Start for loop

{

sensorValue = sensorValue + analogRead(A0); //Add analog values of sensor 500 times

}

sensorValue = sensorValue/500.0; //Take average of readings sensor_volt = sensorValue*(5.0/1023.0); //Convert average to voltage RS_air = ((5.0*1.0)/sensor_volt)-1.0; //Calculate RS in fresh air

R0 = RS_air/3.7; //Calculate R0

Serial.print("R0 = "); //Display "R0" Serial.println(R0); //Display value of R0 delay(1000); //Wait 1 second

}

#### Execution of the Main Program

#include <ESP8266WiFi.h> #include <DHT.h>

#include <ThingSpeak.h>

DHT dht(D5, DHT11);

#define LED_GREEN D2 #define LED_YELLOW D3 #define LED_RED D4 #define MQ_135 A0

int ppm=0;

float m = -0.3376; //Slope float b = 0.7165; //Y-Intercept

float R0 = 3.12; //Sensor Resistance in fresh air from previous code WiFiClient client;

long myChannelNumber = 123456; // Channel id

const char myWriteAPIKey[] = "API_Key";

void setup() {

// put your setup code here, to run once:

Serial.begin(9600); pinMode(LED_GREEN,OUTPUT); pinMode(LED_YELLOW,OUTPUT); pinMode(LED_RED,OUTPUT);

pinMode(MQ_135, INPUT); WiFi.begin("WiFi_Name", "WiFi_Password"); while(WiFi.status() != WL_CONNECTED)

{

delay(200); Serial.print(".");

}

Serial.println();

Serial.println("NodeMCU is connected!"); Serial.println(WiFi.localIP());

dht.begin(); ThingSpeak.begin(client);

}

void loop() {

float sensor_volt; //Define variable for sensor voltage float RS_gas; //Define variable for sensor resistance float ratio; //Define variable for ratio

int sensorValue;//Variable to store the analog values from MQ-135 float h;

float t;

float ppm_log; //Get ppm value in linear scale according to the the ratio value float ppm; //Convert ppm value to log scale

h = dht.readHumidity(); delay(4000);

t = dht.readTemperature(); delay(4000);

sensorValue = analogRead(gas_sensor); //Read analog values of sensor sensor_volt = sensorValue*(5.0/1023.0); //Convert analog values to voltage RS_gas = ((5.0*1.0)/sensor_volt)-1.0; //Get value of RS in a gas

ratio = RS_gas/R0; // Get ratio RS_gas/RS_air

ppm_log = (log10(ratio)-b)/m; //Get ppm value in linear scale according to the ratio value ppm = pow(10, ppm_log); //Convert ppm value to log scale

Serial.println("Temperature: " + (String) t); Serial.println("Humidity: " + (String) h);

Serial.println("Our desired PPM = "+ (String) ppm);

ThingSpeak.writeField(myChannelNumber, 1, t, myWriteAPIKey); delay(20000);

ThingSpeak.writeField(myChannelNumber, 2, h, myWriteAPIKey); delay(20000);

ThingSpeak.writeField(myChannelNumber, 3, ppm, myWriteAPIKey); delay(20000);

if(ppm<=100)

{

digitalWrite(LED_GREEN,HIGH); digitalWrite(LED_YELLOW,LOW); digitalWrite(LED_RED,LOW);

}

else if(ppm<=200)

{

digitalWrite(LED_GREEN,LOW); digitalWrite(LED_YELLOW,HIGH); digitalWrite(LED_RED,LOW);

}

else

{

digitalWrite(LED_GREEN,LOW); digitalWrite(LED_YELLOW,LOW); digitalWrite(LED_RED,HIGH);

}

delay(2000);}

# Chapter 5 RESULTS

The working of the designed prototype has been investigated for the 5 sets of experiments as

described in the following sections

#### EXPERIMENT 1:

Aim: To demonstrate the working of the system in a warm and humid outdoor atmosphere.

Experimental Condition: The experiment was performed on a warm sunny day in a local outdoor area.

###### Observations in ThingSpeak Cloud:

Fig: 5.1 Observations for Experiment 1

###### Setup:

Fig: 5.2 Setup for Experiment 1

Conclusion: We have taken the reference from the Samsung mobile weather app for verifying the values. It matched with a +1.20 error with the temperature data, +5 error with the humidity data and +0.11 error with the PPM data. Hence, we can conclude that the setup has measured the temperature and humidity around the setup area successfully.

#### EXPERIMENT 2:

Aim: To demonstrate the working of the system in the presence of alcoholic gases.

Experimental Condition: The experiment was performed indoor in the presence of alcoholic gases. Drops of an alcoholic mixture (hand sanitiser) were used to produce alcoholic vapours.

###### Observations in ThingSpeak Cloud:

Setup:

Fig: 5.3 Observations for Experiment 2

Drops of Alcoholic

Solution

###### Conclusion:

Fig: 5.4 Setup for Experiment 2

We can observe from the results that the presence of alcohol vapours near the setup can be easily detected by the system. We have taken the reference from the Samsung mobile weather app for verifying the values. It matched with a +1.30 error with the temperature data, +5 error with the humidity data and +0.25 error with the PPM data. Hence, it can be concluded that we can detect the presence of alcoholic vapours with the help of this monitoring system.

#### EXPERIMENT 3:

Aim: To demonstrate the working of the system in smoky conditions.

Experimental Condition: The experiment was performed in the presence of smoke coming from an incense stick placed near the setup.

###### Observations in ThinkSpeak Cloud

Setup:

Fig: 5.5 Observations for Experiment 3

Ignited Incense Stick

Fig: 5.6 Setup for Experiment 3

###### Conclusion:

We can observe from the results that the presence of smoke near the setup can be easily detected by the system. We have taken the reference from the Samsung mobile weather app for verifying the values. It matched with a +1.80 error with the temperature data, +4 error with the humidity data and -0.7 error with the PPM data. Hence, it can be concluded that we can detect the presence of smoke with the help of this monitoring system.

#### EXPERIMENT 4:

Aim: To demonstrate the working of the system in a warm and humid outdoor atmosphere.

Experimental Condition: The experiment was performed at night.

###### Observations in ThingSpeak Cloud:

Fig: 5.7 Observations for Experiment 4

###### Setup:

Conclusion:

Fig: 5.8 Setup for Experiment 4

We have taken the reference from the Samsung mobile weather app for verifying the values. It matched with a +1.20 error with the temperature data, +5 error with the humidity data and -

0.08 error with the PPM data. Hence, we can conclude that the setup has measured the temperature and humidity around the setup area successfully.

#### EXPERIMENT 5:

Aim: To demonstrate the working of the system in an air-conditioned indoor atmosphere.

Experimental Condition: The experiment was performed at room temperature.

###### Observations in ThingSpeak Cloud:

Setup:

Conclusion:

Fig: 5.9 Observations for Experiment 5

Fig: 5.10 Setup for Experiment 5

We have taken the reference from the Samsung mobile weather app for verifying the values. It matched with a +0.6 error with the temperature data, +2 error with the humidity data and -0.03 error with the PPM data. Hence, we can conclude that the setup has measured the temperature and humidity around the setup area successfully.

Table 5.1: Experimental Results

| Expt. No. | Temperature (in celsius) |  |  | Humidity (in %) |  |  | Air Quality (in ppm) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Project Reading | Samsung Weather App Reading | Error | Project Reading | Samsung Weather App Reading | Error | Project Reading | Samsung Weather App Reading | Error |
| 1 | 34.2 | 33 | 1.2 | 70 | 65 | 5 | 8.61 | 8.5 | 0.11 |
| 2 | 33.3 | 32 | 1.3 | 70 | 65 | 5 | 42.25 | 42 | 0.25 |
| 3 | 33.8 | 32 | 1.8 | 74 | 70 | 4 | 52.3 | 53 | -0.7 |
| 4 | 34.2 | 33 | 1.2 | 74 | 69 | 5 | 4.26 | 4.34 | -0.08 |
| 5 | 22.6 | 22 | 0.6 | 59 | 57 | 2 | 0.67 | 0.7 | -0.03 |

Chapter 6 CONCLUSION

In this project IoT based on measurement and display of Air Quality Index (AQI), Humidity and Temperature of the atmosphere have been performed. From  the  information  obtained from the project, it is possible to calculate Air Quality in PPM. The disadvantage of the  MQ135 sensor is that specifically it can’t tell the Carbon Monoxide or Carbon Dioxide level    in the atmosphere, but the advantage of MQ135 is that it is able to detect smoke, CO, CO2, NH4, etc harmful gases.

After performing several experiments, it can be easily concluded that the setup is able to measure the air quality in ppm, the temperature in Celsius and humidity in percentage with considerable accuracy. The results obtained from the experiments  are  verified  through  Google data. Moreover, the led indicators help us to detect the air quality level around the setup. However, the project experiences a drawback that is it cannot  measure the ppm values  of the pollutant components separately. This could have been improved by adding gas sensors for different pollutants. But eventually, it would increase the cost of the setup and not be a necessary provision  to monitor the air quality. Since it’s an  IOT-based project,  it will require  a stable internet connection for uploading the data to the ThinkSpeak cloud. Therefore, it is possible to conclude that the designed prototype can be utilized for air quality, humidity and temperature of the surrounding atmosphere successfully.

REFERENCES

[ 1 ]	https://gaslab.com/blogs/articles/carbon-monoxide-levels

[ 2 ]	https://www.instructables.com/Measuring-Humidity-Using-Sensor-DHT11

[ 3 ]	https://pdf1.alldatasheet.com/datasheet-pdf/view/1307647/WINSEN/MQ135.html

[ 4 ]	https://components101.com/development-boards/nodemcu-esp8266-pinout-features- and-datasheet

[ 5 ]	https://www.arduino.cc [ 6 ]	https://thingspeak.com

[ 7 ] Pasha, S. (2016). ThingSpeak based sensing and monitoring system for IoT with Matlab Analysis. International Journal of New Technology and Research, 2(6).

[ 8 ] Kumar, N. S.,  Vuayalakshmi,  B., Prarthana,  R. J., & Shankar, A. (2016, November).  IOT based smart garbage alert system using Arduino UNO. In 2016 IEEE Region 10 Conference (TENCON) (pp. 1028-1034). IEEE.

[ 9 ] IoT based Air Quality monitoring system using MQ135 & MQ7 with Machine Learning analysis by Kinnera Bharath Kumar Sai M.Tech CSE VIT University, Vellore Subhaditya Mukherjee B.Tech CSE VIT University, Vellore Dr. Parveen Sultana H Associate  Professor Department of CSE, VIT University.

[ 10 ] https://www.codrey.com/electronic-circuits/how-to-use-mq-135-gas-sensor

# Appendix

#### PIN DESCRIPTION OF NODEMCU

###### Pinout diagram of the NodeMCU:

Description:

| Pin Category | Name | Description |
| --- | --- | --- |
| Power | Micro-USB, 3.3V, GND, Vin | Micro-USB: NodeMCU can be powered through the USB port 3.3V: Regulated 3.3V can be supplied to this pin to power the board GND: Ground pins Vin: External Power Supply |

| Control Pins | EN, RST | The pin and the button reset the microcontroller |
| --- | --- | --- |
| Analog Pin | A0 | Used to measure analog voltage in the range of 0-3.3V |
| GPIO Pins | GPIO1 to GPIO16 | NodeMCU has 16 general purpose input-output pins on its board |
| SPI Pins | SD1, CMD, SD0, CLK | NodeMCU has four pins available for SPI communication. |
| UART Pins | TXD0, RXD0, TXD2, RXD2 | NodeMCU has two UART interfaces, UART0 (RXD0 & TXD0) and UART1 (RXD1 & TXD1). UART1 is used to upload the firmware/program. |
| I2C Pins |  | NodeMCU has I2C functionality support but due to the internal functionality of these pins, you have to find which pin is I2C. |

  - DESCRIPTION OF SOFTWARE LIBRARIES USED

ESP8226WiFi Library

The ESP8266WiFi library provides a wide collection of C++ methods (functions) and properties to configure and operate an ESP8266 module.

Commands used are as follows:

- WiFi.begin(“ WiFi Name”, “WiFiPassword”);  Command to connect with WiFi network.

- WiFi.status();  To check the status of the connection. If it returns – WL_CONNECTED  WiFi is connected

If it returns – WL_IDLE_STATUS  WiFi is connected but no internet found If it returns – WL_CONNECT_FAILED  WiFi is not connected

###### DHT11 sensor Library

The DHT sensor library provides a wide collection of C++ methods (functions) and properties to configure and operate the DHT11 sensor module.

The commands used are as follows:

- DHT dht(D5, DHT11);  Set the pin for reading data.

- dht.begin(); Command to connect with DHT11 sensor module.

- dht.readTemperature();  Returns the value of the temperature in Celsius.

- dht.readHumidity();  Returns the value of humidity in percentage.

###### ThinkSpeak Library

The ThinkSpeak library provides a wide collection of C++ methods (functions) and properties to configure and operate the ThinkSpeak cloud.

The commands used are as follows:

	ThingSpeak.writeField(myChannelNumber, 1, t, myWriteAPIKey);  To upload data in the ThinkSpeak Field.

#### COST ESTIMATION OF THE PROJECT

For making the project we have used the following components (as mentioned in Table 2). As per the pricing on the online websites for electronic components, we have formulated a cost estimation.

Table 2: Cost Estimation of the Project

| Components | Price (in Rs) |
| --- | --- |
| NodeMCU V3 | 288 |
| DHT11 Sensor Module | 120 |
| MQ135 Gas Sensor Module | 135 |
| Connecting Wires | 60 |
| LEDs (Red, Green & Yellow) | 9 |
| AC-DC Power Adapter | 120 |
| Female PCB Berg Terminal and cable | 80 |
| Veroboard | 100 |
| Breadboard | 70 |
| Total | 982 |